# Homework for Chapter 5 (and 2.2.4)

- Assume a 1 MB memory.

  - What is the necessary address size if the memory is to be byte-addressable?

  - What is the word size if the address size is 18 bits?

- Given an address size of 16 bits, how big can be the memory:

  - If byte-addressable

  - If the word size is 16 bits

- An instruction set architecture has a 1MB memory of 32-bit words and 16 registers.  An instruction can have at most 1 memory operand and 1 register operand. An instruction must also fit into one word. How many different opcodes (not expanding) can be provided?

- Design an expanding opcode to allow all the following to be encoded in a 36-bit instruction:

7 instructions with two 15-bit addresses and one 3-bit register number

500 instructions with one 15-bit address and one 3-bit register number

40 instructions with no addresses or registers

- Assume a machine with 16-bit instructions and 6-bit addresses. Some instructions have one address and others have two. If there n two-address instructions, what is the maximum number of one-address instructions?

- Consider the following code for the digits 0 through 3:

0: 00000

1: 01111

2: 10011

3: 11100

How many single-bit errors can this code detect? How many can it correct?

- A sequence of octal digits is transmitted redundantly by sending each digit twice, i.e., a 0 is sent as 000 000, a 1 is sent as 001 001, a 6 is sent as 110 110, etc. How many single-bit errors can this code detect? What is the resulting overhead?

- Construct the Hamming code to encode the digits 0 though 3.

Hint:  3 parity bits (position 1, 2, and 4) and 2 data bits (positions 3 and 5) are needed. Fill in the data digits first then determine and fill in the parity bits.

- Construct the Hamming code to encode the digits 0 through F.

- Correct the errors in the following Hamming code words:  0101000, 1111000